Chapter 1

Operation Process Chart

# 1   Introduction

# Charting and diagramming techniques are particularly useful for analyzing a work process because they graphically illustrate and summarize the activities in that process. In this module, we’ll discuss several important charting and diagramming techniques used in methods engineering and operations analysis.

# 1.1   Overview of Charting and Diagramming Techniques

Charting and diagramming techniques are useful for graphically displaying relationships among the various entities included in the graphic. Some objectives for using charts and diagrams to study work include:

1. To allow work processes to be communicated and understood more readily.

2. To allow the use of algorithms specifically designed for a diagramming technique.

3. To divide a given work process into its separate and distinct work elements for more

detailed analysis.

4. To provide an illustrative structure for improvements.

5. To represent an existing or proposed work process or method.

## 1.2   How to Create the Chart or Diagram

Any combination of the following techniques can be used to develop a description of the work process that is ultimately used to create a chart or diagram:

1. The analyst works in the area and already knows how the process works. The analyst then develops a graphic of the process and asks others who are familiar with the process to review the graphic.

2. The analyst spends the requisite time to observe and thoroughly understand the process, then develops a chart or diagram of the process, and asks others who are familiar with the process to review the graphic.

3. The analyst conducts a series of one-on-one interviews with people who are closely familiar with the process, then develops a chart or diagram based on those interviews and asks others who are familiar with the process to review the graphics.

4. The analyst arranges for a group meeting with those intimately familiar with the process using a skilled facilitator to draw input from the participants. The analyst develops a chart or diagram based on notes of the group discussion and asks others who are familiar with the process to review the graphic.

# 1.3   Traditional Industrial Engineering Charting and Diagramming Techniques1

This section includes common charting and diagramming techniques used to analyze an existing operation, sequence of operations, or present proposals for new ways of accomplishing the same operations, or design new operations that have not been performed previously. There are a variety of process charts used in methods analysis. One thing these different charts have in common is the symbols used on the charts and the definitions of those symbols. There are five basic symbols as follow:

Operation:

1. An operation is represented with a circle or the capital letter “O.”

2. The operation symbol is used to represent a person doing the work.

3. An operation occurs when an object is intentionally altered in any of its physical or chemical characteristics.

4. It may be assembled or disassembled from another object, or it may be prepared for another operation, transportation, inspection, or storage.

5. An operation also occurs when information is given or received, or when planning or calculating takes place.

Inspection:

1. An inspection is represented with a diamond or the capital letter “I.”

2. An inspection represents a person who is examining an object and comparing it to a standard.

3. An inspection occurs when an object is examined for identification or is verified for quality or quantity in any of its characteristics.

4. Object, data, or information can be inspected.

Transportation:

1. A transportation is represented with an arrow or the capital letter “T.”

2. Transportation occurs when an object is moved from one place to another.

3. Objects, data, and information can be transported.

4. When the movement occurs as a normal part of an operation or inspection, the action is not considered to be transportation.

Delay:

1. A delay is represented with a capital letter “D.”

2. A delay occurs when an object is temporarily set aside.

3. Th object still requires additional processing, but that processing cannot take place at the current time.

4. When the circumstances that caused the delay are resolved, the object will then receive the processing originally scheduled for it.

5. The amount of time for a delay is relatively short, such as seconds, minutes, hours, or sometimes days.

Storage:

1. A storage is represented with a triangle or a capital letter “S.”

2. Storage occurs when an object is intentionally set aside for the purpose of removing it from the mainstream of activities.

3. The purpose of the storage is to protect the product during its period of inactivity, and to prevent its unauthorized use or removal.

4. The amount of time for storage is relatively one, such as days, weeks, months, or sometimes years.

Combined Activity:

1. When two or more activities are performed simultaneously or by the same employee, then the symbols for those activities are combined.

2. The most common example of a combined activity occurs when an employee performs an operation on an object, and then immediately inspects the object to verify the quality of their own work. The symbol for this combined activity would be a circle inside a diamond or the capital letters “OI.”

The purpose of charting is to demonstrate the current “best way” of accomplishing tasks and to analyze a process with scrutiny to improve the process.

## 1.4   Operation Process Chart

An Operation Process Chart is a graphic, symbolic representation of the act of producing a product or providing a service. Its primary focus is on the Operations and Inspections necessary to do the work.

The rules for constructing an Operation Process Chart are as follows:

1. In the Heading, insert the analyst’s name, the date, and the name of the

operation being studied.

2. At the top of the page, include all other additional information, such as the total number

of operations and inspections.

3. If a product is being studied, a simple sketch of the product should be drawn near the

top of the page below the summary information.

4. Only Operations and Inspections are shown on the chart.

5. Operations are represented with a circle.

6. Inspections are represented with a diamond.

7. If an Inspection is done at the same time as an Operation, then a circle is drawn inside

the diamond to represent the combined activity.

8. If the Operations and Inspections are numbered, then the number should be written

inside the circle or diamond.

9. The sequence of activities should be from the top to the bottom of the page in the order

in which they are done.

10. If there are two or more assembly lines that gradually merge, then they should be

shown side-by-side on the chart as they are done.

11. The symbols should be connected with straight lines to show the correct sequence of

activities.

12. If materials enter the process at an Operation, then the material should be written

above a horizontal line that enters the process from the left. The horizontal line may

be above the operation and then connected to the top of the circle, or the horizontal

line may be drawn on the left side of the circle and attached to the right side of the

circle.

13. If multiple materials enter a process at the same activity, each material must be listed

on a separate horizontal line and the line attached to the circle where the material

enters the process.

14. A brief description of each activity must be written on the right side of the symbol.

15. Occasionally, time, frequency, and/or location may also be included.

An example of an Operation Process Chart is presented in Figure 1.1.

21    Return to Cook’s station

Analyst:

Date of Study:

Operator:

Emily’s American Grill

Operation:

Equipment:

Operation Process Chart

Number

Number

Present Method      _______

Proposed Method   _______

Note: Sketches are below or on the back of this page.

Number

Page     1  of  1

Activity

Operations

Inspections

Quantity

Prepare Pecan Waffle, 2 Sausage Patties, and Grits

21

4

Refrigerator, Grill, Waffle Iron, Microwave

Cook

1     Get Patties out of refrigerator

2     Put Patties onto Grill

Two Patties (3)

3     Put Batter into Waffle Iron

Batter

4     Crushed Pecans on top of Batter

11     Plastic Plate on Table

6     Get Bowl from under Table

7     Put Grits into Bowl

12    2 Maple Syrup Packets on Plate

10      Flip Two Patties on Grill

20    Put Plate in To-Go Area

Bowl

Plate

2 Syrup

13    Butter Packet on Plate

Butter

14    Salt Packet on Plate

Salt

17     Patties on Plate &   Inspect

16    Napkin packet on plate

Napkin pkt (12)

C. Pecans (2)

Grits

8     Add Water to Grits, Stir

Water

9     Bowl of Grits in Microwave

15    Pepper Packet on Plate

Pepper

18     Waffle on Plate & Inspect

19     Grits on Plate & Inspect

Plate with

Waffle, Grits,

2 Sausage Patties, and

all Packets

5     Close Waffle Iron & Activate

Figure 1.1. Operation Process Chart example of a fast-food order prepared by a cook.

1.5   Why Operation Process Charts are Useful in the Workplace

Operation process charts are critical tools in industrial engineering for several reasons:

1. Visualization of Processes

- Clarity: They provide a clear and visual representation of the sequence of operations and inspections involved in a manufacturing or service process.

- Simplification: Complex processes are broken down into individual steps, making them easier to understand and analyze.

2. Identification of Inefficiencies

- Bottlenecks: Operation process charts help identify bottlenecks, where delays occur, and resources are underutilized.

- Redundancies: They highlight redundant or non-value-adding steps that can be eliminated to streamline the process.

3. Improvement and Optimization

- Process Improvement: By mapping out the entire process, engineers can spot opportunities for improvement, such as re-sequencing operations or combining steps to reduce cycle time.

- Lean Manufacturing: They are essential in lean manufacturing practices to identify waste and implement continuous improvement strategies.

4. Standardization

- Consistency: Standardizing processes ensures that operations are performed uniformly, leading to consistent quality and performance.

- Training: Operation process charts serve as effective training tools in Standard Operating Procedure (SOP) manuals for new employees, providing a step-by-step guide to performing tasks.

5. Communication

- Cross-Functional Teams: They facilitate communication among cross-functional teams by providing a common understanding of the process.

- Documentation: Serve as documentation for the current state of the process, and are useful for audits and quality management systems.

6. Planning and Scheduling

- Workload Distribution: Help in planning and scheduling by showing the sequence of operations, allowing for effective workload distribution.

- Resource Allocation: Assist in the optimal allocation of resources, such as labor, machinery, and materials, to various operations.

7. Cost Reduction

- Efficiency Gains: By identifying inefficiencies and redundancies, operation process charts contribute to cost reduction through more efficient use of resources.

- Cycle Time Reduction: Streamlining operations leads to reduced cycle times, which in turn lowers production costs and increases throughput.

8. Quality Control

- Inspection Points: They highlight inspection points within the process, ensuring that quality checks are performed at critical stages.

- Defect Reduction: By optimizing processes, the likelihood of defects and rework is reduced, improving overall product quality.

1.6   Summary

Charts play a vital role in work measurement by providing detailed, visual, and analytical tools to understand, communicate, and improve work processes. Charts are used to aid engineers in designing and improving the way goods and services are produced and delivered. Operation process charts are indispensable tools in the field of industrial engineering. They not only help in visualizing and understanding complex processes but also play a crucial role in identifying and eliminating inefficiencies, standardizing operations, improving communication, and ultimately enhancing productivity and quality within a work environment.

==================================================================

## References

[1] Atkins, R.W. Work Measurement and Ergonomics, Grandpappy, Inc., 2019.

## [2] Groover, M.P. Work Systems and the Methods, Measurement, and Management of Work, Upper Saddle River, NJ: Pearson Prentice Hall, 2007.