Chapter 2

Flow Process Chart and Flow Diagram

# 2   Introduction

Work measurement charts are tools used to determine the time required to complete a task or set of tasks. These charts are vital for assessing productivity, planning, and efficiency in a work environment. They include various techniques such as time studies, predetermined motion time systems (PMTS), and work sampling.

# 2.1   Importance of Work Measurement Charts

# There are several reasons why work measurement charts play an integral role in documenting processes and methods in any workplace.

# Establishing Standards:

# They help in setting standard times for tasks, which serve as benchmarks for performance evaluation and labor cost estimation.

# Resource Allocation:

# By understanding the time required for tasks, managers can allocate resources more effectively, ensuring optimal utilization of labor and equipment.

# Identifying Inefficiencies:

# They highlight areas where time is being wasted, allowing for process improvements and better workflow management.

# Capacity Planning:

# Accurate work measurements aid in capacity planning, ensuring that production schedules are realistic and achievable.

# Cost Control:

# By measuring work accurately, organizations can better control labor costs and improve overall profitability.

# 2.2   Flow Process Chart and Flow Diagram

# 2.2.1   Flow Process Chart

# A Flow Process Chart is a graphic symbolic representation of the work performed to manufacture a product or to provide a service. Its primary focus is on all the activities required to do the work. It uses standardized symbols to represent different types of activities such as operations, inspections, storage, delays, and transportation. A Flow Process Chart is extremely useful because it clearly identifies the non-value-added activities so they can be carefully evaluated as to whether they are necessary.

The rules for constructing a Flow Process Chart are as follows:

1. A separate chart should be constructed for each major phase of a complicated process.
2. Additional pages may be added to the chart, if necessary.

# 3. In the Heading, insert the Analyst's Name, the Date, the Name of the Operation being studied, the Operator's Name, and a Description of any Equipment that is being used.
4. Additional information may also be included at the top of the page at the discretion of the analyst.
5. The flow of work should be listed in sequential order from the top to the bottom of the chart.
6. Number each step in sequential order as it is added to the Flow Process Chart.
7. Operations, inspections, transportations, storages, and delays are all shown on the chart.
8. Operations are represented with a Circle or the typed Letter "O." Operations represent work activities.
9. Inspections are represented with a Diamond or the typed Letter "I." Inspections represent decision activities where conformance is verified or when a decision is made.
10. Transportations are represented with an Arrow or the typed Letter "T." Transportations represent movements that are NOT part of an operation or inspection. Distances must be shown for all transportation. Distances may be measured or estimated. The unit of measure must also be shown, such as inches, feet, yards, or miles.
11. Delays are represented with the Symbol D or the typed Letter "D." Delays represent relatively short interruptions in the flow of activities which may be seconds, minutes, hours, or sometimes days.
12. Storages are represented with a Triangle or the typed Letter "S." Storages represent relatively long periods of inactivity which may be days, weeks, months, or sometimes years.

# An example of a Flow Process Chart is presented in Figure 2.1.

Flow Process Chart

# Emily’s American Grill

Date of Study:

Operation:  Prepare Pecan Waffle, Two Sausage Patties, and Grits with Butter

Operator:  Cook

Equipment:  Grill, Waffle Iron, Microwave Oven, Preparation Table

| Step | Distance | Symbol | Description |
| --- | --- | --- | --- |
| 1 | 4 Feet | T | Walk to the refrigerator. |
| 2 |  | O | Open refrigerator. Remove 2 sausage patties. Close refrigerator. |
| 3 | 3 Feet | T | Carry 2 sausage patties to the grill. |
| 4 |  | O | Place 2 sausage patties on the grill. |
| 5 | 2 Feet | T | Walk to the waffle iron. |
| 6 |  | O | Transfer scoop of waffle batter into the waffle iron. |
| 7 |  | O | Sprinkle a handful of crushed pecans on top of the batter. |
| 8 |  | O | Close the waffle iron and activate the timer. |
| 9 | 2 Feet | T | Walk to the preparation table. |
| 10 |  | O | Get plastic bowl from under the table. |
| 11 |  | O | Place scoop of dry grits into the bowl. |
| 12 | 2 Feet | T | Carry the bowl of dry grits to the sink. |
| 13 |  | O | Add water to the dry grits and stir. |
| 14 | 2 Feet | T | Carry the bowl of damp grits to the preparation table. |
| 15 |  | O | Put bowl of damp grits into microwave and activate. |
| 16 |  | O, I | Flip both sausage patties over on the grill and inspect. |
| 17 |  | O | Place a plastic serving plate on the preparation table. |
| 18 |  | O | Place two maple syrup packets on the plate. |
| 19 |  | O | Place one butter packet on the plate. |
| 20 |  | O | Place one salt packet on the plate. |
| 21 |  | O | Place one pepper packet on the plate. |
| 22 |  | O | Place the sealed knife, fork, and napkin package on the plate. |
| 23 | 2 Feet | T | Carry the plate to the grill. |
| 24 |  | O, I | Transfer both cooked sausage patties onto the plate and inspect. |
| 25 | 2 Feet | T | Carry the plate to the waffle iron. |
| 26 |  | D | Wait for the waffle to finish cooking. |
| 27 |  | O, I | Transfer waffle onto plate and inspect. |
| 28 | 2 Feet | T | Carry the plate to the preparation table. |
| 29 |  | D | Wait for the grits to finish cooking. |
| 30 |  | O, I | Place the grits onto the plate and inspect. |
| 31 | 1 Foot | T | Carry plate to the to-go food pickup area. |
| 32 |  | O | Place the plate in the to-go food pickup area |

Figure 2.1. Flow Process Chart example of a fast-food order prepared by a cook.

# 2.2.2   Flow Diagram

# A flow diagram is a visual representation of the physical movement of materials or people through a process. It uses arrows and other symbols to depict the flow paths within a work environment. After a Flow Process Chart has been constructed, a Flow Diagram of the same process can be illustrated on a scale drawing of the work area.

The title "Flow Diagram" should appear at the top of the sketch. The Analyst's name, the Date, and the Name of the Operation should also appear at the top of the diagram. The approximate scale should also be shown somewhere on the sketch.

The same numbering sequence and the same symbols that were used on the Flow Process Chart are also used on the Flow Diagram. Descriptive information is not shown on the Flow Diagram. The Flow Process Chart must be consulted for a description of each corresponding activity.

The activities (operations, inspections, transportations, delays, and storages) should be connected with arrows in the same sequence in which they appear on the Flow Process Chart. In other words, connect activity one to activity two with an arrow. Connect activity two to activity three with an arrow. Continue to connect the activities in sequence until you reach the last activity in the sequence.

# An example of a corresponding Flow Diagram is given in Figure 2.2.

Figure 2.2. Flow Diagram example of a fast-food order prepared by a cook.

# 2.2.3   Linkage Between Flow Process Chart and Flow Diagram

# Complementary Tools:

# While the flow process chart provides a detailed step-by-step breakdown of the process activities, the flow diagram shows the physical movement and layout. Together, they offer a comprehensive view of the process.

# Holistic Analysis:

# Using both tools allows for a holistic analysis of the process, identifying both the sequence of activities and the spatial arrangement, which is crucial for optimizing workflows.

2.3   Why Flow Process Charts and Flow Diagrams are Useful in the Workplace

Flow Process Charts and Flow Diagrams play critical roles in any work environment, whether industrial, agricultural, distribution, logistics, service, or office, etc., for a variety of reasons:

- Process Visualization:

  - They provide a clear and detailed visual representation of the process, making it easier to understand and communicate.

- Identifying Inefficiencies:

  - By mapping out the entire process, these tools help identify inefficiencies, bottlenecks, and areas of waste.

- Improving Workflow:

  - They facilitate the redesign of workflows for improved efficiency and productivity by showing the interaction between different process steps and the movement of materials.

- Training and Standardization:

  - These charts and diagrams are valuable in Standard Operating Procedure (SOP) manuals for training new employees, providing a clear guide to the process steps and physical layout.

- Problem Solving:

  - They are essential tools for problem-solving, allowing engineers to pinpoint specific issues and develop targeted solutions.

- Enhancing Communication:

  - They improve communication among team members and departments by providing a common visual language for discussing processes.

- Supporting Lean Initiatives:

  - These tools are integral to lean manufacturing and continuous improvement initiatives, helping to identify and eliminate waste in the process.

2.6   Summary

Work measurement charts, flow process charts, and flow diagrams are indispensable tools in an industrial engineering context. They play a critical role in understanding, analyzing, and improving work processes, thereby enhancing efficiency, productivity, and overall operational effectiveness in a work environment.

==================================================================

## References

[1] Atkins, R.W. Work Measurement and Ergonomics, Grandpappy, Inc., 2019.

## [2] Groover, M.P. Work Systems and the Methods, Measurement, and Management of Work, Upper Saddle River, NJ: Pearson Prentice Hall, 2007.