# Chapter 11

Predetermined Motion Time System (PMTS)

11   Introduction

Predetermined Motion Time Systems (PMTS) are methods used to set labor standards in a consistent and accurate manner. Work tasks are analyzed by breaking them down into basic motions or work elements. The PMTS systems consist of a database of basic motion elements and their associated normal time values, which are then used to establish standard times. This allows for precise measurement and standardization of work without the need for time-consuming direct time studies. Basic motions include:

- Reach

- Grasp

- Move

- Release

  - PMTS Procedure

The PMTS procedure typically follows four steps, as follows:

Step 1: Synthesize the method, in terms of basic motion elements, that would be used to perform the task.

Step 2: Obtain individual normal time values from the PMTS tables for each motion element based on the work variables and conditions under which the element is performed, and then sum the work element normal times to determine the normal time for the task.

Step 3: Evaluate the method for possible improvements by eliminating motions, reducing distances, and so on.

Step 4: Apply allowance to the total normal time to determine the standard time for the task.

Although several PMTS families exist in the world today, two primary PMTS families are discussed in this chapter: (1) MTM and (2) MOST.

11.2   Methods-Time Measurement (MTM)

MTM is one of the most well-known PMTS, developed by the Methods Engineering Council in the United States. The MTM family consists of several systems, each suited for different types of work environments and complexities. The development and introduction of MTM systems include:

- MTM-1 (1948): The original and most detailed system, suitable for tasks that are

- highly repetitive and involve short cycles. It breaks tasks down into basic motions such as reach, move, turn, grasp, position, and release.

- MTM-2 (1965): A less detailed system compared to MTM-1, designed for less repetitive tasks with longer cycle times. It consolidates some of the basic motions into larger motion groups to simplify the process.

- MTM-3 (1970): An even simpler and faster system than MTM-2, used for tasks with longer cycle times and less precision required in the standard times. It uses broader categories of motions and is easier to apply.

- MTM-V (Variable) (1975): Developed for variable work environments where tasks change frequently. It is less precise but more flexible.

- MTM-SAM (Standard Data) (1980): Used for standard repetitive tasks and provides standard data that can be quickly applied to similar tasks.

Examples of MTM:

- MTM-1: Detailed analysis of an assembly line worker placing parts on a conveyor belt.

- MTM-2: Time measurement for a worker performing machine operations with some manual handling.

- MTM-3: Estimating time for administrative tasks involving document handling and filing.

11.2.1   Differences Between First-Level PMTS and Higher-Level PMTS

First-level PMTS use basic motion elements, such as reach, grasp, release, while higher-level PMTS combine several basic motion elements into motion sequences. First-level PMTS are very detailed, with body motions and conditions specified in their databases. Higher-level PMTS used condensed databases, with fewer body motions specified in the tables and, consequently, longer normal time values for each motion sequence.

11.2.2   Motion Aggregates

A motion aggregate combines several basic motion elements that are performed together in a motion sequence. An example is “lay an object down,” which consists of the basic motions reach and release.

11.2.3   Time Units in Methods-Time Measurement

The normal time values in the MTM tables are in TMUs (or time measurement units). One TMU = 0.00001 hr = 0.036 sec, as follows.

1 TMU=0.00001 hr=0.0006 min=0.036 sec                     (11.1)

Conversely,

1 sec=27.8 TMU                                               (11.2)

11.2.4   Advantages of Higher-Level PMTS

Advantages of higher-level PMTS over lower-level PMTS include:

- Faster application speed (i.e., ratio of the time required to determine a time standard relative to the time standard itself). For example, an application speed ratio of 50 means it takes 50 minutes of a time analyst’s time to determine a 1-minute time standard. There is an inverse relationship between application speed ratio and the accuracy of the time standard. The level of detail required to compute normal times in TMUs for MTM-1, for example, relates to a slow application speed ratio, but the normal times are more accurate than for higher-level MTMs with motion aggregates.

- Can be used for longer feasible cycle times, and, as a result,

- Are easier to apply.

11.2.5   MTM Procedure

MTM is a procedure that analyzes any manual operation or method into the basic motions required to perform it, represented by an MTM symbol, and assigns a predetermined TMU time standard from basic motion element tables to each task. Table values are determined by the nature of the motion and the conditions under which the motions are made. The total TMU time can then be converted to seconds by multiplying the total TMUs by 0.036 sec/TMU. Note that the MTM table values are “normalized” time values. Hence, there is no need to assign performance ratings to an MTM symbol. The MTM-1 tables (1a-1k)1 follow.

Table 11.1(a). Normal Time Values for MTM-1 Motion Element: Reach (R)

|  |  | Time in TMU |  |  |  |  |  | Case and Description |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance |  |  |  |  |  | Hand in Motion |  |  |
| cm | inches | A | B | C or D | E | A | B | A Reach to object in fixed location, or to object in other hand or on which other hand rests. |
| <2.0 2.5 5.1 7.6 10.1 12.5 15.2 17.8 20.3 22.9 25.4 30.5 35.6 40.6 45.7 50.8 55.9 61.0 66.0 71.1 76.2 | < 0.75 1 2 3 4 5 6 7 8 9 10 12 14 16 18 20 22 24 26 28 30 | 2.0 2.5 4.0 5.3 6.1 6.5 7.0 7.4 7.9 8.3 8.7 9.6 10.5 11.4 12.3 13.1 14.0 14.9 15.8 16.7 17.5 | 2.0 2.5 4.0 5.3 6.4 7.8 8.6 9.3 10.1 10.8 11.5 12.9 14.4 15.8 17.2 18.6 20.1 21.5 22.9 24.4 25.8 | 2.0 3.6 5.9 7.3 8.4 9.4 10.1 10.8 11.5 12.2 12.9 14.2 15.6 17.0 18.4 19.8 21.2 22.5 23.9 25.3 26.7 | 2.0 2.4 3.8 5.3 6.8 7.4 8.0 8.7 9.3 9.9 10.5 11.8 13.0 14.2 15.5 16.7 18.0 19.2 20.4 21.7 22.9 | 1.6 2.3 3.5 4.5 4.9 5.3 5.7 6.1 6.5 6.9 7.3 8.1 8.9 9.7 10.5 11.3 12.1 12.9 13.7 14.5 15.3 | 1.6 2.3 2.7 3.6 4.3 5.0 5.7 6.5 7.2 7.9 8.6 10.1 11.5 12.9 14.4 15.8 17.3 18.8 20.2 21.7 23.2 |  |
|  |  |  |  |  |  |  |  | B Reach to single object in location that may vary slightly from cycle to cycle. |
|  |  |  |  |  |  |  |  | C Reach to object jumbled with other objects in a group so that search and select occur. |
|  |  |  |  |  |  |  |  | D Reach to a very small object or where accurate grasp is required. |
|  |  |  |  |  |  |  |  | E Reach to indefinite location to get hand in position for body balance or next motion or out the way. |
| Additional |  | 0.4 | 0.7 | 0.7 | 0.6 | TMU per 2.54 cm> 76 cm (per 1.0 in> 30 in.) |  |  |

Table 11.1(b). Normal Time Values for MTM-1 Motion Element: Grasp (G)

| Type of Grasp | Case | Time, TMU | Description and Object Dimensions |  |
| --- | --- | --- | --- | --- |
| Pickup | lA lB | 2.0 3.5 | Any size object, by itself Object very small or lying close against a flat surface |  |
|  | lCl 1C2 1C3 | 7.3 8.7 10.8 | Interference with grasp on bottom and one side of cylindrical object | Diameter> 1.3 cm (0.5 in.) Diameter 0.6 to 1.3 cm (0.25 to 0.5 in.) Diameter< 0.6 cm (0.25 in.) |
| Regrasp | 2 | 5.6 | Change grasp without relinquishing control |  |
| Transfer | 3 | 5.6 | Control transferred from one hand to other |  |
| Select | 4A | 7.3 | Object jumbled with other objects so that search and select occur | Size larger than 2.5 x 2.5 x 2.5 cm (1 X 1 X 1 in.) 0.6 x .6 x .3 cm (.25 x .25 x .12 in) to 2.5 x 2.5 x 2.5 cm (1 x 1 x 1 in.) Size smaller than .6 x .6 x .3 cm (.25 X .25 X .12 in.) |
|  | 4B | 9.1 |  |  |
|  | 4C | 12.9 |  |  |
| Contact | 5 | 0 | Contact, sliding, or hook grasp |  |

Table 11.1(c). Normal Time Values for MTM-1 Motion Element: Move (M)

|  |  | Time in TMU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distance |  |  |  |  | Hand in motion | Weight up to | Formula Parameters |  | Case and Description |
| cm | inches | A | B | C | B | kg (lb) | Constant | Factor |  |
| <2.0 | < 0.75 | 2.0 | 2.0 | 2.0 | 1.7 | 1.1 (2.5) 3.4 (7.5) 5.7 (12.5) 7.9 (17.5) 10.2 (22.5) 12.5 (27.5) 14.7 (32.5) 17.0 (37.5) 19.3 (42.5) 21.5 (47.5) | 0 2.2 3.9 5.6 7.4 9.1 10.8 12.5 14.3 16.0 | 1.00 1.06 1.11 1.17 1.22 1.28 1.33 1.39 1.44 1.50 | A Move object to |
| 2.5 | 1 | 2.5 | 2.9 | 3.4 | 2.3 |  |  |  | other hand or |
| 5.1 | 2 | 3.6 | 4.6 | 5.2 | 2.9 |  |  |  | against stop. |
| 7.6 | 3 | 4.9 | 5.7 | 6.7 | 3.6 |  |  |  |  |
| 10.1 | 4 | 6.1 | 6.9 | 8.0 | 4.3 |  |  |  |  |
|  |  |  |  |  |  |  |  |  | B Move object to approximate or indefinite location. |
| 12.5 | 5 | 7.3 | 8.0 | 9.2 | 5.0 |  |  |  |  |
| 15.2 | 6 | 8.1 | 8.9 | 10.3 | 5.7 |  |  |  |  |
| 17.8 | 7 | 8.9 | 9.7 | 11.1 | 6.5 |  |  |  |  |
| 20.3 | 8 | 9.7 | 10.6 | 11.8 | 7.2 |  |  |  |  |
| 22.9 |  |  |  |  |  |  |  |  | C Move object to exact location. |
|  | 9 | 10.5 | 11.5 | 12.7 | 7.9 |  |  |  |  |
| 25.4 | 10 | 11.3 | 12.2 | 13.5 | 8.6 |  |  |  |  |
| 30.5 | 12 | 12.9 | 13.4 | 15.2 | 10.0 |  |  |  |  |
| 35.6 | 14 | 14.4 | 14.6 | 16.9 | 11.4 |  |  |  |  |
| 40.6 | 16 | 16.0 | 15.8 | 18.7 | 12.8 |  |  |  |  |
| 45.7 | 18 | 17.6 | 17.0 | 20.4 | 14.2 |  |  |  |  |
| 50.8 | 20 | 19.2 | 18.2 | 22.1 | 15.6 |  |  |  |  |
| 55.9 | 22 | 20.8 | 19.4 | 23.8 | 17.0 |  |  |  |  |
| 61.0 | 24 | 22.4 | 20.6 | 25.5 | 18.4 |  |  |  |  |
| 66.0 | 26 | 24.0 | 21.8 | 27.3 | 19.8 |  |  |  |  |
| 71.1 | 28 | 25.5 | 23.1 | 29.0 | 21.2 |  |  |  |  |
| 76.2 | 30 | 27.1 | 24.3 | 30.7 | 22.7 |  |  |  |  |
| Additional |  | 0.8 | 0.6 | 0.85 | TMU per 2.54 cm> 76 cm (per 1.0 in.> 30 in.) |  |  |  |  |

Table 11.1(d). Normal Time Values for MTM-1 Motion Element: Position (P)

|  |  |  | TimeinTMU |  |
| --- | --- | --- | --- | --- |
| Class | Description of Fit | Symmetry | Easy to Handle | Difficult to Handle |
| 1 | Loose (no pressure required) | sss | 5.6 9.1 | 11.2 14.7 |
|  |  | NS | 10.4 | 16.0 |
| 2 | Close (light pressure required) | s ss | 16.2 19.7 | 21.8 25.3 |
|  |  | NS | 21.0 | 26.6 |
| 3 | Exact (heavy pressure required) | s ss | 43.0 46.5 | 48.6 52.1 |
|  |  | NS | 47.8 | 53.4 |
| Key: S = symmetrical, SS = semi-symmetrical, NS = nonsymmetrical. |  |  |  |  |

Table 11.1(e). Normal Time Values for MTM-1 Motion Element: Release (RL)

| Case | TimeinTMU | Description |
| --- | --- | --- |
| 1 | 2.0 | Normal release performed by opening fingers as an independent motion |
| 2 | 0 | Contact release with no finger motion |

Table 11.1(f). Normal Time Values for MTM-1 Motion Element: Disengage (D)

|  |  |  | Time in TMU |  |
| --- | --- | --- | --- | --- |
| Class | Description of Fit | Height of Recoil | Easy to Handle | Difficult to Handle |
| 1 | Loose (very slight effort, blends with subsequent move) Close (normal effort, slight recoil) (1 to 5 in) Tight (considerable effort, hand recoils markedly) | Up to 2.5 cm (1 in) | 4.0 | 5.7 |
| 2 |  | 2.5 to 12.7 cm |  |  |
|  |  |  | 7.5 | 11.8 |
| 3 |  | 12.7 to 30 cm | 22.9 | 34.7 |
|  |  | (5 to 12 in) |  |  |

Table 11.1(g). Normal Time Values for MTM-1 Motion Element: Turn (T)

|  | Time in TMU for Degrees Turned |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weight, kg (lb) | 30° | 45° | 60° | 75° | 90° | 105° | 120° | 135° | 150° | 165° | 180° |
| Small, up to 0.9 (2) | 2.8 | 3.5 | 4.1 | 4.8 | 5.4 | 6.1 | 6.8 | 7.4 | 8.1 | 8.7 | 9.4 |
| Medium,1 to 4.5 (2 to 10) | 4.4 | 5.5 | 6.5 | 7.5 | 8.5 | 9.6 | 10.6 | 11.6 | 12.7 | 13.7 | 14.8 |
| Large, 4.5 to 16 (10 to 35) | 8.4 | 10.5 | 12.3 | 14.4 | 16.2 | 18.3 | 20.4 | 22.2 | 24.3 | 26.1 | 28.2 |

Table 11.1(h). Normal Time Values for MTM-1 Motion Element: Apply Pressure (AP)

| Symbol | TimeinTMU | Description |
| --- | --- | --- |
| APA | 10.6 | Apply pressure alone |
| APB | 16.2 | Apply pressure preceded by regrasp |

Table 11.1(i). Normal Time Values for MTM-1 Motion Element: Eye Travel (ET) and Eye Focus (EF)

| Eye motion | Symbol | TimeinTMU | Key to Symbols |
| --- | --- | --- | --- |
| Eye travel | ET | 15.2L D | L = distance between points from and to which eye travels, D = perpendicular distance from the eye to the line of travel. Maximum time allowed = |
|  |  |  | 20TMU |
| Eye focus Reading | EF (none) | 7.3 5.05N | N = number of words read (330 words/min) |

Table 11.1(j). Normal Time Values for MTM-1 Motion Element: Body, leg, and foot motions (various

symbols given in table)

| Motion | Symbol | TimeinTMU | Description and Conditions |
| --- | --- | --- | --- |
| Sit Stand Turn body Turn body Bend Stoop Arise Arise Kneel Kneel Arise Arise | SIT STD TBCl TBC2 B S AB AS KOK KBK AKOK AKBK | 34.7 43.4 18.6 37.2 29.0 29.0 31.9 31.9 29.0 69.4 31.9 76.7 | From standing position From seated position Turn body 45° to 90°, Case 1 - Lagging foot not aligned with leading foot Turn body 45° to 90°, Case 2 - Lagging foot aligned with leading foot Bend body forward so hands can reach knees Stoop body forward so hands can reach floor Arise from bent position Arise from stooped position Kneel on one knee Kneel on both knees Arise from kneeling position on one knee Arise from kneeling position on both knees |
| Walk Walk Walk Leg motion Leg motion | WXFT WNP WNPO LM6 LMX | 5.3 per ft 15.0/pace 17.0/pace 7.1 7.1 + l.2(X-6) | Walking in ft of distance, X = distance in ft Walking in number of paces, N = number of paces Walking in number of paces with weight or obstruction, N = number of paces Move leg up to 6 in. any direction Move leg more than 6 in. any direction, where X = distance of movement |
| Foot motion Foot motion | FM FMP | 8.5 19.1 | Foot moves up to 4 in. hinged at ankle Foot moves up to 4 in. hinged at ankle, apply heavy pressure with leg muscles |

Table 11.1(k).	MTM-1 Simultaneous Hand and Arm Motion Elements

| Motion of One Hand |  | Reach |  |  | Grasp |  |  | Move |  |  | Position |  |  | Disengage |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Motion of other hand | Case or Class | A E | B | C D | lA 2 5 | lB lC | 4 | A Bma | B | C | 1S lSS lNS 2S 2SS 2NS |  |  | lE 1D | 2 |
| Reach | A,E B C,D | 1 1 1 | 1 1 1 | 1 1 1 | 1 1 1 | 1 1 2 | 1 2 3 | 1 1 2 | 1 1 2 | 2 2 3 | 1 2 3 | 1 2 3 | 2 3 3 | 1 1 2 | 1 1 3 |
| Grasp | lA, 2, 5 lB,lC 4 | 1 1 1 | 1 1 2 | 1 2 3 | 1 1 1 | 1 3 2 | 1 2 3 | 1 1 1 | 1 1 2 | 1 2 3 | 1 3 3 | 1 3 3 | 3 3 3 | 1 3 3 | 3 3 3 |
| Move | A B C | 1 1 2 | 1 1 2 | 2 2 3 | 1 1 1 | 1 1 2 | 1 2 3 | 1 1 1 | 1 1 1 | 1 1 2 | 1 2 3 | 1 2 3 | 2 3 3 | 1 1 2 | 1 1 3 |
| Position | 1S | 1 1 2 | 2 2 3 | 3 3 3 | 1 1 3 | 3 3 3 | 3 3 3 | 1 1 2 | 2 2 3 | 3 3 3 | 2 3 3 | 3 3 3 | 3 3 3 | 3 3 3 | 3 3 3 |
| lSS,2S lNS,2SS,2NS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Disengage | 1E,1D 2 | 1 1 | 1 1 | 2 3 | 1 3 | 3 3 | 3 3 | 1 1 | 1 1 | 2 3 | 3 3 | 3 3 | 3 3 | 1 1 | 1 1 |

3Bm is Case B with hand in motion.	(continued)

Table 11.1(k).	(continued)

Key:The cell numbers indicate the degree of difficulty when motions are performed simultaneously. 1 = Easy to perform simultaneously. Use the longest motion element time. 2 = Can be performed simultaneously with practice. Use the longest motion element time. 3 = Difficult to perform simultaneously. Add the times of the two simultaneous motion elements.

Assumptions: All Reach, Grasp, and Move motions are performed within the area of normal vision. In the Position and Disengage motion elements, objects are assumed easy to handle. In general, the degree of difficulty increases if these assumptions are violated.

Motions not included in the table: Turn: normally degree of difficulty = 1 except when Turn is controlled or with Disengage. Position Class 3: degree of difficulty = 3. Disengage Class 3: normally, degree of difficulty = 3. Release: degree of difficulty = 1.

Example 1.   MTM-1 (REACH)

A worker at a workbench performs a REACH. The sought-after wrench is lying on top of the workbench, and the distance of the reach is 12 in. Determine the MTM-1 symbol and normal time in both TMUs and seconds for this motion element.

Solution:

MTM-1 Symbol=R12A

Tn=9.6 TMU0.036 secTMU=0.3456 sec

Example 2. MTM-1 (MOVE)

An office worker sitting at her desk performs a MOVE. The file folder being moved weighs less than one lb. It is moved to an exact location in her file cabinet a distance of 30 in. Determine the MTM-1 symbol and normal time in both TMUs and seconds for this motion element.

Solution:

MTM-1 Symbol=M20C1

Tn=0+130.7=30.7 TMU0.036 secTMU=1.1052 sec

Example 3. MTM-1 (MOVE)

A production worker standing at her machine performs a MOVE. The part being moved weighs less than 1 lb. It is moved to an approximate location in her work area a distance of 4 ft. Determine the MTM-1 symbol and normal time in both TMUs and seconds for this motion element.

Solution:

MTM-1 Symbol=M48B1

Given a distance of 4 ft=48 in, we have 24.3+0.618=35.1 TMU

Hence, Tn=0+1(35.1)=35.1 TMU0.036 secTMU=1.2636 sec

Example 4. MTM-1 (Walking)

A worker must walk a distance of 20 ft (both ways) to perform a certain manual task. What is the MTM-1 normal time in both TMUs and seconds for this motion element?

Solution:

MTM-1 Symbol=W40FT

Tn=405.3=212 TMU0.036 secTMU=7.632 sec

Example 5. MTM-1 (Sequence of motions)

A work element in a setup task consists of the following MTM-1 elements: (1) Reach for a long T-handle Allen wrench located 20 in away hanging on a hook on a machine; (2) Grab the T-handle. It weighs less than 1 lb.; (3) Move the T-handle to tighten a loose nut on a head a distance of 20 in.; (4) Position the T-handle into the nut head with close fit, non-symmetrical, easy-to-handle; (5) Tighten nut on the head turning the T-handle four turns of 180 degrees; (6) Move the T-handle back to the hook on the machine; (7) Position the T-handle on the hook, loose fit, non-symmetrical, easy to handle; (8) Release T-handle normally. What is the MTM-1 normal time in both TMUs and seconds for this motion element?

Solution:

1 MTM-1 Symbol=R20A, Tn=13.1 TMU

2 MTM-1 Symbol=G1A, Tn=2.0 TMU

3 MTM-1 Symbol=M20C, Tn=0+1.022.1=22.1 TMU

4 MTM-1 Symbol=P2NSE, Tn=21.0 TMU

5 MTM-1 Symbol=TS180, Tn=49.4=37.6 TMU

6 MTM-1 Symbol=M20C, Tn=0+1.022.1=22.1 TMU

7 MTM-1 Symbol=P1NSE, Tn=5.6 TMU

8 MTM-1 Symbol=RL1, Tn=2.0 TMU

Total Tn=125.5 TMU0.036 secTMU=4.518 sec

Example 6. MTM-1 (Sequence of Motions)

A worker picks up a small bolt from a bin of similar bolts on top of a machine and places it into a bolt hole on a cutting die to secure it to a cylinder on the machine. The bin is within 12 in from her starting position. She grasps the bolt in the bin and moves the bolt from the bin to the bolt hole on the cutting die a distance of 20 in. She positions the bolt accurately in the bolt hole with a close fit and light pressure, easy to handle, non-symmetrical. She turns the bolt by hand 180o six times. She then performs a normal release of the bolt. What is the MTM-1 normal time in both TMUs and seconds for this motion element?

Solution:

1 MTM-1 Symbol=R12C, Tn=14.2 TMU

2 MTM-1 Symbol=G1A, Tn=2.0 TMU

3 MTM-1 Symbol=M20B2, Tn=0+1.015.6=15.6 TMU

4 MTM-1 Symbol=P2NSE, Tn=21.0 TMU

5 MTM-1 Symbol=TS180, Tn=69.4=56.4 TMU

6 MTM-1 Symbol=RL1, Tn=2.0 TMU

Total Tn=111.2 TMU0.036 secTMU=4.00 sec

11.3   Maynard Operation Sequence Technique (MOST)

MOST is another widely used predetermined motion time system developed by Zjell Zandin in the early 1980s to analyze and measure work activities. MOST is designed to be more efficient and faster to apply than traditional MTM systems. It is less detailed than MTM-1 but provides a balance between accuracy and efficiency in time study analysis. MOST breaks down tasks into sequence models, each comprising standard sequences of movements and corresponding time values. It simplifies the process by focusing on the sequence of movements rather than individual motions, which significantly reduces the time required to establish a standard.

MOST analyzes work in terms of activity sequences, with each sequence consisting of standard phases or activities. The three main types of MOST are:

- BasicMOST: The most commonly used version, suitable for general tasks. It uses four activity sequence models: General Move, Controlled Move, Tool Use, and Manual Crane. In this chapter, we will focus on General Move, Controlled Move, and Tool Use.

- MiniMOST: A more detailed version for short-cycle, highly repetitive tasks, primarily used in manufacturing environments.

- MaxiMOST: Designed for long-cycle, non-repetitive tasks, typically used in construction, maintenance, and similar industries.

Examples of MOST:

- BasicMOST: Evaluating the time required for a worker to move a tool from one location to another and use it in an assembly operation.

- MiniMOST: Analyzing repetitive tasks on an assembly line where workers perform the same set of movements repeatedly.

- MaxiMOST: Determining the time for a construction worker to complete various stages of a building project, such as laying bricks or installing electrical wiring.

11.3.1   Basic MOST

Basic MOST is focused on work that involves the movement of objects (i.e., parts, tools, inventory) from one location to another in the workplace. Basic MOST uses motion aggregates, or activity sequence models, which combine several basic motion elements that are performed together in an activity sequence.

11.3.1.1   MOST Procedure

MOST is a procedure that analyzes any manual operation or method into the basic motions required to perform it in an activity sequence model and assigns a predetermined time standard from activity sequence model tables to each task. Table values are determined by the nature of the motion and the conditions under which the motions are made. Index values from the tables are shown as subscripts for each letter in the activity sequence model. The index values are summed, and this total is multiplied by 10 to yield the total task time, in TMUs, which can then be converted to seconds or minutes. Note that the MOST table values are “normalized” time values. Hence, there is no need to assign performance ratings to an activity sequence model. Figure 11.1 displays symbols and descriptions used to describe General Move, Controlled Move, and Tool Use.

Figure 11.1. Symbols and descriptions used for activity sequence models.

11.3.1.2   Activity Sequence Models

The activity sequence model for a General Move is illustrated below.

The MOST table for a General Move is shown in Figure 11.2. Here, A = action distance, B = type of body motion, C = type of gained control, and P = placement of object.

As with all activity sequence models, the index value used to describe each aspect of the activity sequence model becomes a subscript for that letter. The index values are summed and then multiplied by 10 to yield the normal time, in TMUs. The total TMUs, when multiplied by 0.036 sec/TMU will yield the total normal time for the task.

Figure 11.2. MOST table for General Move.

Example 7. General Move

An office worker walks eight steps, bending down to pick up a file folder that fell to the floor, arising, returning the eight steps, and placing the file folder loosely on her desk. What is the MOST normal time in both TMUs and seconds for this activity sequence model?

Solution:

A  B  G  A  B  P  A

A16B6G1A16B0P1A0=

16+6+1+16+0+1+0=40*10=400 TMU

Tn=400 TMU0.036 secTMU=14.4 sec

Example 8. General Move

A shipping worker slightly bends over and reaches for a box that weighs less than 1 lb that is within reach on a table to put it on a shelf 3 steps away and returns to her original position. What is the MOST normal time in both TMUs and seconds for this activity sequence model?

Solution:

A  B  G  A  B  P  A

A1B3G1A6B0P1A1=

1+3+1+6+0+1+1=13*10=130 TMU

Tn=130 TMU0.036 secTMU=4.68 sec

The activity sequence model for a Controlled Move is illustrated below

The MOST table for a Controlled Move is shown in Figure 11.3. The activity sequence ABG is described in the General Move table. Here, M = type of controlled move, X = process time (in sec, min, or hr), and I = type of alignment.

Figure 11.3. MOST table for Controlled Move.

Example 9. Controlled Move

A bandsaw operator takes one step back to gain control of a small handful of 4 x 8 ft sheet stock to move it 16 in. The sheet is aligned to a long, affixed guide on one side of the sheet stock. It is not necessary to reposition the operator’s hands during alignment. What is the MOST normal time in both TMUs and seconds for this motion sequence?

Solution:

A  B  G  M  X  I  A

A3B0G3M6X0I1A0=

3+0+0+6+0+1+0=10*10=100 TMU

Tn=100 TMU0.036 secTMU=3.6 sec

Example 10. Controlled Move

A worker picks up a small bolt from a bin of similar bolts on top of a machine and places it into a bolt hole on a cutting die to secure it to a cylinder on the machine. The bin is within 12 in from her starting position. She grasps the bolt in the bin and moves the bolt from the bin to the bolt hole on the cutting die a distance of 20 in. She positions the bolt accurately in the bolt hole with a close fit and light pressure, easy to handle, non-symmetrical. She turns the bolt by hand 180o six times, which takes 3 sec. She then performs a normal release of the bolt. What is the MOST normal time in both TMUs and seconds for this activity sequence model?

Solution:

A  B  G  M  X  I  A

A1B0G1M6X10I10A0=

1+0+1+6+10+10+0=28*10=280 TMU

Tn=280 TMU0.036 secTMU=10.08 sec

The activity sequence model for a Tool Use is illustrated below

Return

The various types of tools are denoted by letter designation in Figure 11.4 and described in Table 11.2.

Figure 11.4. MOST Tool Use Code Letters

where

Table 11.2. Symbols and Descriptions for Tool Use.

The MOST table for Tool Use is shown in Figure 11.5(a) and (b). The activity sequence ABG is described in the General Move table and the activity sequence model for ABP is described in the Controlled Move table. Here, * is designated for the type of tool used.

Figure 11.5(a). MOST table for Tool Use.

Figure 11.5(b). MOST table for Tool Use (cont’d).

Example 11. Tool Use

A mechanic picks up a feeler gauge from her rolling toolbox five steps away and checks the gap width of a sparkplug on a gas-powered mobile cart and then puts the feeler gauge back in her toolbox. What is the MOST normal time in both TMUs and seconds for this activity sequence model?

Solution:

A  B  G  A  B  P *  A  B  P  A

A10B0G1A10B0P6M24A10B0P1A10=

10+0+1+10+0+6+24+10+0+1+10=72*10=720 TMU

Tn=720 TMU0.036 secTMU=25.92 sec

Example 12. Tool Use

A worker picks up a screw from her workbench, positions it into a drilled hole, and turns it four spins with her fingers. She then picks up a screwdriver from the workbench, positions it on the head of the screw, fastens the screw with four turns, and then lays the screwdriver aside. What is the MOST normal time in both TMUs and seconds for this activity sequence model?

Solution:

This requires two activity sequence models, corresponding to the two sentences of the work description.

A  B  G  A  B  P *  A  B  P  A

The first activity sequence model:

A1B0G1A0B0P3F10A0B0P0A0=15*10=150 TMU

The second activity sequence model:

A1B0G1A1B0P3F10A1B0P1A0=18*10=180 TMU

Tn=150 TMU+180 TMU=330 TMU0.036 secTMU=11.88 sec

11.3.2   MOST Software

MOST is available in software packages to expedite the computation of activity sequence models. An example of input values using MOST software3 is shown in Figure 11.6.

Figure 11.6. An example of MOST software.

11.4   Summary

Predetermined Motion Time Systems (PMTS), such as the MTM family and MOST, provide standardized ways to measure and improve worker productivity by breaking down tasks into basic motion elements or activity sequences, assigning standard times in TMUs which can be converted into seconds, and thus allowing for better planning and optimization of work processes.

==================================================================

## References

## [1] Groover, M.P. Work Systems and the Methods, Measurement, and Management of Work, Upper Saddle

## River, NJ: Pearson Prentice Hall, 2007.

## [2] Atkins, R.W. Work Measurement and Ergonomics, Grandpappy, Inc., 2019.

## [3] Freivalds, A. Niebel’s Methods, Standards, and Work Design, 13th ed., McGraw-Hill, 2014.