Tanker Simulation Model

Tankers arrive periodically at a harbor and discharge their cargo into shore tanks. When a shore tank is full, or nearly so, its contents are automatically transferred to the refinery. While this transfer is taking place, a shore tank may not be filled by a tanker.

Model data

Timings in hours:

Tanker arrival rate negexp: 0.125/hour

Setup time for pump constant: 0.5 hours

Pumping rate constant: 1000 tons/hour

Discharge rate constant: 4000 tons/hour

Capacities in 1000 ton units:

Tanker loads: 15 .. 25 equally likely

Shore tank volume: 70

Run the simulation for 1000 continuous hours with 5 shore tanks. Take as initial conditions that two shore tanks are empty and free, one is currently discharging and will be free at 8 hours, and that the other two are currently being loaded and will be freed at times 12 (with 45 units still free) and 3.5 (with 25 units free) respectively. The first tanker arrives at time 0.0.

Tanker simulation emhancements

As an extension to the basic model, we assume that tankers now carry one of five types of oil. The grade of oil is indicated by integer type which takes values in the range 1 through 5, a higher value indicating a better grade of oil. In order to unload when docked, they need a shore tank which (in order of preference):

- has the same type of fuel and sufficient capacity to take all that the tanker is carrying

- has the same type of oil but not enough capacity to accept the full load

- is empty and previously held either the same or a better type of fuel

- is empty and previously held an inferior type of oil. In this case, the shore tank must first be cleaned. This takes a long time and is the last resort.

In this new situation it is much harder to express the search by using find, although it could perhaps be done by devious use of priority within class shoretank. Even so it would lead to a non-transparent program text and in cases like this we really need another mechanism so that we can express the program logic clearly. It is best to have all the decisions at one point in the program instead of scattered over several entities. This is where the hitherto unexplained boolean procedure avail local to waitq (see figure 5.3) comes into its own. A call Q.avail(E, condition); [ ref(waitq) Q;] tries to locate an entity E satisfying the condition (just like find), but does not coopt the entity E if found nor block the caller if an entity E satisfying the condition cannot be found. It returns true and sets E to refer to the found entity if one can be located; if not, it returns false and sets E to none.

In this variation, it is convenient to use two waitqs to hold available shore tanks. We place completely empty shore tanks in EMPTYQ and partly full shore tanks in TANKQ. Note that in this description, a shore tank will discharge its contents into the refinery when less than 5 units of volume are available.