Problem 1 (25 pt.): Pascal has poured one cup of water into the thin metal tube that was about 10m tall and the sides of the barrel leaked.

- (10 pt.) What is the additional pressure exerted by water in the tube?

- (10 pt.) Find the additional force on the barrel side wall due to the water in the tube. Barrel total heigh h=1.5m, R=0.5m, assuming cylindrical share: Aside wall=2πRh

- (5 pt. +15 bonus pt.) Find the total force on barrel side wall including the pressure from water in the barrel itself. Hint – you can break this up into two parts: force from pressure in the tube (reuse result from part b) and force from the pressure of the water in the barrel itself.

Problem 2 (25 pt.): A solid aluminum cylinder of  V=100cm3 is submerged in water. ρAl=2.7gcm3

V1

(10 pt.) Find the total weight of this cylinder in water.

- (15 pt.) Find the volume V1 of the empty cavity (assume density of 0) in this cylinder to make it float just barely (at least, not sink). Total V is the same.

Bonus (10 pt.) The cavity V1 from b is now increased by 10%. Assuming it’s placed uniform within the cylinder, how much of the cylinder will be submerged? Give the answer as % of total V.

A1

A2

Air intake

Problem 3 (25 pt.): A design of the aerator in the faucet is very simple – it uses the liquid flow to create the mixture of water and air and save on the amount of water used. It can be schematically represented by two tubes of areas A1 and A2, and the opening for the air.

- (10 pt.) First, assume only A1 is there. The water is under the pressure of 1.5 atm when coming to A1 with almost 0 velocity (hint - use 1 atm as 105Pa) from the left, and right of the tube is open to air. What is the velocity of the water exiting the tube A1?

- (5 pt.) If A1=1.5A2, what is the velocity of water in A2? Use result from part a for velocity in A1.

- (10 pt.) What is the pressure in A2?

Bonus (5 pt.) What will happen to air in the intake because of the pressure from part c?

Problem 4 (25 pt.):

A car tire has pressure of 2.5 atm at 10oC. What is the new pressure, if the volume is the same and the temperature increases to 78oF?

Bonus (10 pt.) How many mols of air are inside, if the air density is about 1 kg/m3 and molar mass is 29 g/mol . Tire volume is 1.5∙10-2m3