Problem 1: In a hydraulic press, a small piston has the area A1=20cm2 and the large piston: A2=1m2

- (3 pt) A person of m=60kg applies all their weight at the end of the handle that is distance d from the pivot. What is the force pushing on the small piston if its 0.1d from the pivot point?

- (2pt) What is the pressure that the small piston creates in the oil (assume oil is static)

- (3 pt) What is the force that the large piston exerts on the object? *use Pascal’s principle*

Problem 2: Water is flowing at a rate of 2m3s  through a tube with a diameter of 1m.

- (2 pt) Show that flow rate Q=vA (hint – write out what v is.)

- (2 pt) What is the speed of the water in the tube?

- (3 pt) What is the speed of the water after the tube narrows to a diameter of 0.5m

- Bonus (5) If the initial pressure is 80kPa, what is the pressure of the water after the tube narrows to a diameter of 0.5m? There is no change in height.